Section 611.7904  Reporting and Recordkeeping Requirements

Suppliers required to sample must report to the Agency according to the timeframes and provisions of Section 611.840 and retain records according to the provisions in Section 611.860.

a)	Suppliers must report the information from initial monitoring specified in the following table:

Section 611.7904(a) Table 1:  Data to Report from Initial Monitoring

| If you are a . . . | You must report . . . |
| --- | --- |
| Supplier monitoring for regulated PFAS under the requirements of Section 611.7902(b)(1) on a quarterly basis. | 1. All sample results, including the locations, number of samples taken at each location, dates, and concentrations reported. 2. Whether a trigger level, defined in Section 611.7902(a)(5), was met or exceeded in any samples. |
| Supplier monitoring for regulated PFAS under the requirements of Section 611.7902(b)(1) less frequently than quarterly. | 1. All sample results, including the locations, number of samples taken at each location, dates, and concentrations reported. 2. Whether a trigger level, defined in Section 611.7902(a)(5), was met or exceeded in any samples. |

b)	Suppliers must report the information collected during the compliance monitoring period specified in the following table:

Section 611.7904(b) Table 1:  Data to Report from Compliance Monitoring

| If you are a . . . | You must report . . . |
| --- | --- |
| Supplier monitoring for regulated PFAS under the requirements of Section 611.7902(b)(2) on a quarterly basis. | 1. All sample results, including the locations, number of samples taken at each location, dates, and concentrations during the previous quarter. 2. The running annual average at each sampling point of all compliance samples. 3. Whether a trigger level, defined in Section 611.7902(a)(5), was met or exceeded in any samples. 4. Whether an MCL for a regulated PFAS in Section 611.311(c) was met or exceeded in any samples. 5. Whether, based on Section 611.7903, an MCL was violated. |
| Supplier monitoring for regulated PFAS under the requirements of Section 611.7902(b)(2) less frequently than quarterly. | 1. All sample results, including the locations, number of samples taken at each location, dates, and concentrations during the previous monitoring period. 2. Whether a trigger level, defined in Section 611.7902(a)(5), was met or exceeded in any samples. 3. Whether an MCL for a regulated PFAS in Section 611.311(c) was met or exceeded in any samples. 4. Whether, based on Section 611.7903, an MCL was violated (e.g., the results from a single sample are more than 4 times the MCL). |

BOARD NOTE:  This Section derives from 40 CFR 141.904

(Source:  Added at 50 Ill. Reg. 2531, effective February 17, 2026)